# System Description & Requirements (SDR)

Title: JNET2 Transport Networking SDR

SDR #: JNET2 – 001

Version: 001-001A

Author: Mark Altshuller

Date: Dec. 2019

History Revisions

| Version | Author | General Version Content | Issue Date |
| --- | --- | --- | --- |
| 001-001A | Mark Altshuller | Initial Draft | TBD |

Document Approval

| Name | Title | Approved/Not Approved | Comments |
| --- | --- | --- | --- |
| Alex Bilchinsky | CTO | TBD |  |

Contents

1	Introduction	7

1.1	General JNET2 Network Architecture Description	7

1.2	JNET2 Network Overview & High Level Requirements	11

1.3	JNET2 Transport Networking SDR Scope	13

2	Target Audience	15

3	System Requirements Summary	16

4	General Feature Description & Scope	17

5	Functional Requirements – JNET2 Transport Multi-Domain Networking	18

5.1	JNET2 VLAN based concept description	18

5.2	Functional Requirement – F0001, JNET2 Node generic intra-node architecture	21

5.2.1	Functional Requirement – F0002, Internal MAC Addressing structure	24

5.2.2	Functional Requirement – F0003, Internal IP Addressing structure	25

5.2.3	Functional Requirement – F0004, Internal UDP/TCP Port structure and communication principals	26

5.3	Functional Requirement – F0005, JNET2 Node initial setup procedure	26

5.4	Functional Requirement – F0005, End-to-End downstream traffic forwarding principals	29

5.4.1	Requirement – F0006, POP/CAP Node E2E Downstream Packet Forwarding Requirement	30

5.4.2	Requirement – F0007, Transit Node E2E Downstream Packet Forwarding Requirement	34

5.4.3	Requirement – F0008, Termination Node E2E Downstream Packet Forwarding Requirement	34

5.5	Functional Requirement – F0009, End-to-End upstream traffic forwarding requirement	36

5.6	Functional Requirement – F0010, Peer-to-peer traffic forwarding requirement	37

5.7	Requirement – F00011, Broadcast Traffic Requirements	38

5.8	Requirement – F00012, DHCP Support Requirements	39

5.9	Requirement – F00013, 802.1p TOS field support requirement	41

5.10	Requirement – F00013, Direct inter-node communication	42

5.11	Requirement – F00014, JNET2 Technician Direct Communication Requirement	43

5.12	F00015 - JNET2 Security X.509 / TLS certificate-based support requirements	44

6	Management Requirements	47

6.1	F00016 - JNET2 Management Architecture - Networking Structure Requirement	47

6.2	Configuration Requirements	48

6.3	Fault, Alarm, Event Requirements	48

6.4	Counters / KPI requirements	48

7	Performance Requirements	49

7.1	Performance Requirement – P0001, [TITLE]	49

8	Hard Ware Requirements	49

8.1	HW Requirement – H0001, [TITLE]	49

9	Feature Inter-Dependency Requirements	49

9.1	Dependency Requirement – D0001, [TITLE]	49

10	References	50

11	Glossary	51

Figure List

Figure 1: Juganu "foam" system - general view	7

Figure 2: Bubble Principal Structure	8

Figure 3: Basic JNET2 Node Connectivity in Bubble	8

Figure 4: Typical Light Poles Street Structure	9

Figure 5: JNET2 P3 Beam/Antenna Switching Topology	9

Figure 6: POP JNET2 Node 2-Parent radio connectivity for P3 system	10

Figure 7: P1/P2 JNET2 System Star & Mixed Topologies	11

Figure 8: JNET2 Transport Generic Multi-Domain Architecture	11

Figure 9: Peer-to-peer traffic use case with JNET2 transport domains	13

Figure 10: MEF Ethernet Services	18

Figure 11: JNET2 transport VLAN QinQ stack	19

Figure 12: Outer VLAN ID base forwarding concept within JNET2 bubble	20

Figure 13: Outer VLAN ID structure	20

Figure 14: JNET2 Node Networking Communication Node Diagram	21

Figure 15: Single-Domain JNET2 Node Networking Communication Node Diagram	23

Figure 16: JNET2 Node Networking Initialization Process for Multi IP-Domain	27

Figure 17:JNET2 Node Networking Initialization Process for Single IP-Domain	28

Figure 18: Principal E2E downstream packet flow	29

Figure 19: POP/CAP Node e2e downstream packet flow	30

Figure 20: Downstream e2e Packet Flow in Transit Node	34

Figure 21: Termination Node e2e downstream packet flow	35

Figure 22: E2E Upstream packet flow in the JNET2 Bubble	36

Figure 23: Peer-to-Peer traffic flow diagram within the Bubble	37

Figure 24: Initial DHCP Procedure example originated by the JNET2/Foam IP Host	39

Figure 25: Forwarding Tables update with DHCP sniffing	41

Figure 26: TOS/Priority Field within 802.1Q VLAN header	41

Figure 27: Direct Communication Protocol Stack	43

Figure 28: JNET2 TLS/X509 Security Scenarios	44

Figure 29: EAP/TLS Procedure for 802.11	45

Figure 30: X.509 Certificate Format	46

Figure 31: JNET2 Node Management Connectivity Diagram	47

Table List

Table 1: JNET2 Node MAC addressing	24

Table 2: IEEE OUI and CID assignment	25

Table 3: JNET2 Node IP addressing	25

Table 4: JNET2 Node Blocks UDP/TCP port numbering	26

Table 5: E2E Downstream Packet flow steps	30

Table 6: External Tunnel mapping to SVLAN	31

Table 7: POP Function Internal JNET2 Bubble ARP Table	32

Table 8: POP Function Transport Proxy ARP Table	33

Table 9: JNET2 Node Transport Function DL forwarding table	34

Table 10: Termination Forwarding Table	35

Table 11: Upstream e2e packet flow functional steps	37

Table 12: Peer-to-Peer packet flow functional steps	38

Table 13: DHCP Procedure Support Requirements within the Bubble	40

Table 14: X.509 Certificate Fields Provisioning Requirements for JNET2 Node and Technician Terminal	46

# Introduction

# General JNET2 Network Architecture Description

JNET2 system targets to enable broadband connectivity between the Street Lamp Nodes (called JNET2 Nodes) in order to provide variety of the use cases and service. The Figure 1 presents the overall Juganu “foam” system vision

Figure 1: Juganu "foam" system - general view

“Foam” System specifies the overall Juganu solution for variety of the applications, while JNET2 transport solution provides the connectivity layer to enable all applications served by the “foam” system to communicate with the external world and between them. The examples of the applications shown in the  Figure 1.

The main JNET2 transport solution value is providing the cost-efficient alternative to other transport solution options, such as fiber, DOCSIS or other costly wireless transport solutions (e.g. dedicated radios such as microwaves, mmWave, etc.). Using integrated radios JNET2 broadband transport solution enables highest density (every 30-50m) for the connected applications (such as cameras, WiFi Access Points, Sensors, LTE small cells) creating unique “foam” paradigm. Actually, JNET2 transport is just an enabler to build variety of “smart networks” with highest density in very attractive location in the cities (above the streets). [TBD – marketing text explaining the “foam” use cases]

The foam system deployment structure defines “Bubble” as a basic structure. Bubble is defined as a group of JNET2 Nodes (Street Lamp Nodes), while one of the JNET2 nodes functions as the POP (Point of Presence) to the transport network (see Figure 2).

As shown in Figure 2 the POP JNET2 Node used as a traffic root of the bubble tree topology and as a such defined as Hierarchy 00. Hierarchical tree structure of the bubble is a basic concept of the JNET2 network.

The target JNET2 system (called P3) aims to achieve ~2gbps throughput per BUBBLE, while each JNET2 Node within the bubble may consume as peak traffic 1Gbps (even more) and average traffic per JNET2 Node should be the “bubble throughput” divided by number of Nodes per Bubble. For example, if number of Nodes per Bubble is 200 and the “bubble throughput” is 2gbps, therefore the average traffic per JNET2 Node is 10mbps (while the peak rate per Node is 1Gbps).  As per traffic model the Bubble may serve 100s of JNET2/foam nodes.

Figure 2: Bubble Principal Structure

In order to achieve high performance JNET2 transport solution, each JNET2 Node comprises of two 802.11ac/ax radios operating on 5Ghz unlicensed band (in future 6Ghz planned). One of the radios is used as AP mode (called “Parent”) while other is used as STA mode (called “Child”). The Figure 3 shows the basic connectivity approach within the bubble. The Parent is connected to the Child of the lower tree hierarchy.

Figure 3: Basic JNET2 Node Connectivity in Bubble

Since the typical street lighting structure considers that lamp poles are located along the streets (see Figure 4), therefore in order to achieve hundreds lamp poles connected to the same Bubble and assuming bad 5Ghz band signal propagation characteristics, the tree topology should enable deep multi-hop network (e.g. 20 and more hops / hierarchies)

Figure 4: Typical Light Poles Street Structure

In such deep multi-hop network, while each hop is to the neighboring Lamp poles (distance of 30-50 meters), JNET2 P3 system uses Sector/Beam switching technology (see Figure 5) in order to reduce intra-system interference effect.

Figure 5: JNET2 P3 Beam/Antenna Switching Topology

As mentioned above JNET2 Node comprises two 802.11 5Ghz radios, which can be used as following:

- In any JNET2 Node: one STA (Child) and one AP (Parent)

- In POP JNET2 Node (for P3 only with Sectorized/Beam-Steering antenna): two APs (Parent). Thus, enabling to connect 2 trees / bubbles via single POP JNET2 (see Figure 6).

Figure 6: POP JNET2 Node 2-Parent radio connectivity for P3 system

As part of the JNET2 system roadmap evolution Juganu develops two solutions generations:

- JNET2 P3 – the target system with sectorized antennas and smart scheduling algorithms in order to provide deep multi-hop transport network within the bubble. This system aims to cover citywide deployments of thousands Lamp Poles. The goal of the P3 system is minimizing the requirement for number of POPs in the network. If each Bubble may serve about 200 JNET2 Nodes, the overall two-Bubble area connected to the single PoP Node comprises of 400 Nodes, which may cover 400,000 m2 (0.4 km2) [calculation example: 400 lamp poles – each one covers area of 1000 m2)

- JNET2 P1/P2 - it is the JNET2 system early generation, which targets smaller bubble sizes and the main motivation is enabling short time-to-market (TTM) solution. As P3, P1/P2 system comprises of two 5Ghz 802.11 radios, but the main difference is OMNI antennas vs Sectorized/Beamswitching antennas in P3. Since OMNI antennas create a lot of intra-system interference it is difficult to achieve high throughput bubbles with many Nodes and deep multi-hop topology. Therefore P1/P2 target topology is Star or Mixed (as shown in Figure 7). Such topologies can be more efficient with intra-system interference management, while number of hops (in Mixed topology) is low. Star and Mixed topologies can be considered as private cases of the Tree Topology; therefore, many hierarchical topology aspects can be shared with P3 system. From the marketing perspective P1/P2 system targets few 10s JNET2 Nodes per Bubble and can be used for deployments in “Campus” size areas.

As can be seen from Figure 7 the major difference between P1/P2 star&mixed topologies vs P3 deep-tree topology that P1/P2 system is more efficient when using high parent:child ratio (e.g. 1:15 and more) and low number of hops, while P3 system targets low parent:child ratio (e.g. 1:4) and high number of hops.

Figure 7: P1/P2 JNET2 System Star & Mixed Topologies

# JNET2 Network Overview & High Level Requirements

The general JNET2 transport network structure is shown in Figure 8.

Figure 8: JNET2 Transport Generic Multi-Domain Architecture

As can be seen from Figure 8 each JNET2 Node comprises various application entities/hosts (e.g. cameras, public WiFi, LTE network, …). The JNET2 transport network/layer should enable each application entity separate Networking Domain. It is also shown that JNET2 transport network itself is a Networking Domain as well. Actually, the Network Multi-Domains’ approach provides generic concept to separate Business / Operational entities one from each other while running on top of the JNET2 transport network.

In order to explain the business & operational meaning behind the Network Domains refer to the following use case example:

[Use Case]

JNET2 transport network belongs to the Municipality IT department. IT department provides connectivity services to various applications physically located within the Juganu “foam” Node (Street Lamp):

- Cameras & Video Analytics system - managed by the Municipality Safe City Department or other Public Safety private/governmental entity

- Public WiFi service - managed & operated by WiFi Service Provider

- Private WiFi (e.g. police) -  managed & operated by the Police Department

- LTE Network -  managed & operated by LTE Service Provider

- Connected car network (e.g. RSU) - managed & operated by “Traffic Governmental” entity

- Water meters – managed & operated by Water Supply Company

- Electricity meters & smart grid control – managed & operated by Electricity Company

Providing the above transport connectivity services to the different companies, Municipality IT department may charge for the connectivity service and should be capable to enable connectivity service agreement per each.

[end of use case]

Typical Networking Domain comprises complete separation of:

- IP address management– meaning each domain may manage own IP space independently to other Networking Domains within the JNET2 system. For example, it results that each networking domain maintains own DHCP server. Such independent IP address space control enables the Network Domain owner to configure and enable the connectivity to any Servers within the network and outside the network per own business interests and policies

- Authentication / Authorization – for instance, each domain enables to maintain independent subscription database, accounting system, etc

- Management System – each networking domain enables independent system for configuration, fault, alarming and other management operations.

Important to mention that all above capabilities are completely transparent to the JNET2 Transport Network Operator.

Figure 8 shows that traffic is running over JNET2 transport network end-to-end (e2e) in both directions, meaning each IP-packet within the domain is transformed from the “Service Provider”/Operational Entity to the appropriated destination JNET2 Node Application and similarly in the opposite direction.

In addition JNET2 transport system should enable peer-to-peer traffic within each transport domain, but avoiding any communication between the domains. Figure 9 shows the examples for peer-to-peer services JNET2 transport should enable. In this example Cameras and RSUs belong to one domain, while WiFi AP belong to another domain. Within the transport domain each Application Entity may send IP-packets to another Application Entity. The packets are switched within the JNET2 Bubble, without the need going forward & back via the Domain Core Network. Peer-to-peer functionality improves the overall latency and saves JNET2 throughput. Throughput and latency are scared resources within the JNET2 Transport network.

Figure 9: Peer-to-peer traffic use case with JNET2 transport domains

The Peer-to-Peer intra-domain traffic is switched using MAC addresses, therefore practically each Transport Domain is the Ethernet Local Access Network (LAN) and all Application Entities (or Hosts) within the JNET2 Bubble (and possibly for group of Bubbles) belong to the same IP Sub-Net. From that reason the number of Hosts per LAN domain per Bubble is limited. For example for Public WiFi service the Users’ traffic is forwarded as e2e traffic, but the intra-WiFi APs control traffic (e.g. mobility events) can be switched as peer-to-peer within the same Domain. Peer-to-peer service is unique transport capability and therefore should be coordinated with the Domain Service/Operational Entity.

NOTE: Important to mention that JNET2 transport solution allows single Transport Domain use case (the private case of the Multi-Domain architecture). Single Transport Domain means that JNET2 transport and all Application Hosts are managed and operated by the same Service/Operational Entities (e.g. Municipality)

# JNET2 Transport Networking SDR Scope

This SDR includes the following requirement scope:

- JNET2 Node internal connectivity and addressing structure (e.g. MAC, IP addresses, Node ID)

- VLAN based architecture

- JNET2 CAP/POP Node function – this part specifies interfacing with different external transport options (e.g. L2TP, MPLS, VLAN, …)

- JNET2 Node functional requirements to enable transport communication with Management Server

- JNET2 Node functional requirements for e2e traffic forwarding (both directions), peer-to-peer and broadcast traffics on the domain basis

  - JNET2 CAP/POP Node

  - JNET2 End Point Node (Destination / Origin)

  - JNET2 Transit Node

- DHCP support specific requirements

- JNET2 ad hoc connectivity requirement – refers to the ad hoc direct communication between neighboring JNET2 nodes independent to the Bubble topology. The possible needs for such communication can be the following: information exchange during topology discovery, topology changes, special future features

- Local Technician connectivity requirement – enabling the technician to connect to the specific JNET2 Node in the street through local communication (e.g. via Bluetooth, WiFi AP)

- 802.1p transport QoS requirements

- Security requirements enabling JNET2 Node authentication, traffic encryption

The SDR scope does not include the following JNET2 requirement specifications, which are defined in other JNET2 SDRs:

- JNET2 Topology discovery, establishment, changes, keep-alive – this SDR assumes that tree topology is already established. Of course, it is inter-independency between the Bubble topology creation, optimization, changes processes and networking processes. For example, the Bubble topology change impacts on the forwarding tables

- JNET2 MAC Scheduling – this SDR assumes that scheduled provides all required services. In general this SDR is agnostic of the scheduler type and can be applicable for any.

This spec is relevant for all JNET2 phases (P1, P2 & P3), but in some sections this document emphasizes the specific requirement on per P-basis.

# Target Audience

[This document is targeted for internal use only. The target audience is Juganu R&D persons, Product Management, Customer Support and any other technology personal, who is exposed to JNET2 system. In addition, the document can be used by Juganu technology partners pending to the approval by CTO/Architect].

# System Requirements Summary

[The following table is used as the compliance table for R&D to confirm the support of the specific feature requirements. As final result all Priority 1 requirements should be approved by R&D after agreement on the functionality. The requirements can be confirmed on the release basis].

| Requirement # | Requirement Description | Priority (1, 2, 3) | Target Release | R&D confirmation status (Yes, No, Pending) | Comment |
| --- | --- | --- | --- | --- | --- |
| JNET2-001-F0001 |  |  |  |  |  |
| JNET2-001-M0001 |  |  |  |  |  |
| JNET2-001-P0001 |  |  |  |  |  |
| JNET2-001-H0001 |  |  |  |  |  |
| JNET2-001-D0001 |  |  |  |  |  |

Requirement Numbering Structure

Requirement Type:

- F – Functional Requirement: the requirement related to the functional capabilities of the system

- M – Management Requirement: specifies requirements related to the configuration & provisioning, counters, alarming, management events and etc. related to the feature

- P – Performance Requirements

- H – Hardware Requirements: specific requirements for the HW to support the feature

- D – Dependency Requirements: Defines inter-dependency requirements with other features in the system

# General Feature Description & Scope

[To provide the feature marketing & system background (executive summary style – 1-3 pages) The reader should understand the general feature functionality from this Chapter. This description can be used for the Product System Description documentation, delivered to the customer, therefore need to avoid including any confidential information]

[General Scope of the next chapters and short description of each chapter coverage]

# Functional Requirements – JNET2 Transport Multi-Domain Networking

[ Functional requirements section is the main part of the SDR. The author should explain all capabilities and requirements of the feature, which will be summarized as the System Requirements in the Section #3]

[The author should provide the initial assumptions (marketing, system, feasibility), which resulted to the specific feature concept]

[The author should explain different options for the solution and explain the reason to select the specific system requirement solution]

[Each System Requirement should be explained in the sub-titles of this Section]

# JNET2 VLAN based concept description

In order to enable multi-domain e2e & peer-to-peer traffic functionality VLAN QinQ (reference to IEEE802.1ad) approach was selected. Major motivations behind VLAN selection as a basic concept for Multi-Domain Transport solution are:

- MEF (Metro Ethernet Forum) oriented architecture (refer to Figure 10) – actually JNET2 transport is the infrastructure segment between the “Operator” and the “Application End-Points” which enables all MEF services as shown in the figure. Since MEF compliancy is common for network infrastructure, JNET2 transport network may easily to be integrated.

Figure 10: MEF Ethernet Services

- Standard based Ethernet framework, which is commonly supported by industrial L2 switches

- VLAN header is commonly used to support Ethernet traffic priority based on 802.1p tagging. All transport equipment support 802.1p including mapping between L3 DSCP TOS (Type Of Service) and 802.1p COS (Class Of Service) for e2e QOS.

- Easy integration between the outside Transport and JNET2 transport segment. VLAN tagging is commonly supported by the transport routers / switches.

- VLAN is connectionless traffic tunneling, therefore fits the wireless unstable environment

- VLAN enables point-to-point, point-to-multi-point, mesh and broadcast traffic management using either VLAN tagging and MAC addresses switching. Thus, enabling required flexibility for JNET2 transport to support e2e, peer-to-peer and broadcast traffic in multi-domain environment

- Reducing the Service Provider transport configuration complexity. With VLAN solution each Service Provider Transport Domain is exposed with single transport tunnel per Bubble (equivalent to MEF E-LAN or E-Tree services) vs e2e tunnel per each node (equivalent to MEF E-Line Service)

- To minimize switching processing time in the JNET2 transit node. Since JNET2 Bubble is a tree topology with many hops, majority of the traffic is a transit traffic.

The Figure 11 shows the protocol stack concept for VLAN tagging within the JNET2 transport

Figure 11: JNET2 transport VLAN QinQ stack

- JNET2 Transport layer consists of 802.11 MAC (Bridge mode/WDS) or Ethernet Header and Outer VLAN tag. Outer VLAN tag is internal for the Bubble JNET2 transport and not exposed outside (NOTE: in future it can be considered Outer VLAN tagging extension to the Globbule -FFS). Outer VLAN tagging is uniquely identifies the JNET2 Node within the Bubble (see Figure 12). As per current concept there is one-to-one relation between JNET2 MAC address (related as Node infrastructure MAC address – explained below) and Outer VLAN ID (VID). The JNET2 End points (POP JNET2 Node and Target/Origin Application Node) should add the Outer VLAN ID based on the destination MAC address: Outer VID mapping (more details below). The transit traffic is switched based on Outer VID (more details below). The main motivation why using Outer VLAN ID and not the Node MAC address is explained below. In future it is possible that multiple infrastructure MAC addresses can be mapped to the single Outer VLAN ID, which is possibly additional motivation to use single ID (VID) per JNET2 Node.

Figure 12: Outer VLAN ID base forwarding concept within JNET2 bubble

The Outer VLAN ID has the following structure (see Figure 13), which is defined for the specific JNET2 traffic management purposes. As can be seen in the figure that VLAN ID field within the VLAN Tag has two sub-fields (Juganu specific)

- Juganu Packet Switch Type Indicator (PSTI) – PSTI indicates the Node about the traffic type to optimize the processing time when forwarding the packets in the deep tree topology (the specifics are explained in the next sections)

- JNET2 Node ID (10 bits) – unique JNET2 Node identificatory within the Bubble. For example, if the JNET2 changes the association and goes to another Bubble most probably it will accept different JNET2 NODE ID (future feature).

Figure 13: Outer VLAN ID structure

- Service transport layer – it is represented by Inner VLAN ID (also called as Service VLAN – SVLAN). SVLAN represents the Service Provider transport tunneling within the JNET2 transport segment. Actually, the different IP Domains per each Service Provider are separated by SVLAN IDs. The northbound tunneling options (between the POP/CAP JNET2 Node and external transport infrastructure) can be different, but POP JNET2 Node is responsible to make 1:1 mapping between external tunnels and SVLANs. It is possible that nearby transport router will be configured with relevant SVLAN ID to avoid any tunneling mapping within JNET2 POP Node.

- Service/Application/IP layer – Services and Applications on top of IP layer are completely transparent to the JNET2 transport layer. Meaning, the Service Provider may manage tunneling over IP layer (for example GTP or SCTP in 4G/5G networks) transparently as e2e or peer-to-peer traffic within the JNET2 bubble. However, since SVLAN is equivalent to Ethernet LAN, the JNET2 transport should function as MAC based L2 switching to enable IP packets to be forwarded to the proper destination (JNET2 Node). The JNET2 Nodes should support ARP tables (IP to MAC mapping) per SVLAN. The broadcast packets (e.g. ARP, DHCP) are broadcasted within the SVLAN and forwarded to the appropriated transport tunnel by POP JNET2 Node (more details below – it can be different behavior for broadcast packets in e2e and peer-to-peer SVLAN modes).

# Functional Requirement – F0001, JNET2 Node generic intra-node architecture

The Figure 13 presents JNET2 Node internal network block diagram and the logical communication interfaces between different Application Hosts, Application Processor and Networking Processor within the  JNET2 Node.

Figure 14: JNET2 Node Networking Communication Node Diagram

The following requirements should be supported per each block in the diagram. The JNET2 Node consist of:

- Network Processor – used as the Node networking switch providing all networking services connecting between JNET2 transport and internal Application hosts. Networking Processor communicates with all peripherals over Ethernet as shown in Figure 13. Networking Processor should support different types of packet forwarding capabilities and functions (see more details below) as following:

  - MAC switching

  - VLAN tag switching (SVLAN & Outer VLAN) and coloring

  - UDP/TCP Port switching

  - TCP/IP stack (currently only IPv4)

  - DHCP Client

  - DHCP Server (used for local IP addresses allocation and for Direct Inter-Node Communication as explained in Section ‎5.10)

  - Additional functionalities as POP/CAP JNET2 Node (details in Section ‎5.4.1)

  - 802.1p Class Of Service support (QoS) - queuing

- Application Processor – from the networking perspective Application Processor provides communication switching service connecting IP world with non-IP Application Hosts (e.g. MIPI Cameras, Sensors, …). Each non-IP Application Host interface (and some related application instance – e.g. Camera edge analytics) is represented by Application Processor as combination of SVLAN/ IP Address / TCP or UDP Source Port.   Application Processor should support the following functions:

  - Mapping in both directions SVLAN/IP Address/TCP or UDP port to the Application host instance / physical port: e.g. MIPI, UART

  - TCP/IP stack (currently only IPv4)

  - DHCP Client (Optional ???)

  - Management embedded client per domain. In case of the single domain, Network Processor is representing the management clients (e.g. bridging) within the Application Domain (assumed that each management client is responsible for type of the sensor connected to the Application Domain – e.g. Camera, …). The Management Architecture is TBD (part of other SDR).

  - 802.1p Class Of Service support (QoS) coloring

NOTE: As shown in Figure 13 either Network Processor and Application Processor logically divided to the slices. Each logical slice represents Networking IP Domain, which is identified by the SVLAN ID.

- 802.11 5GHz radios – two radios used for JNET2 transport communication between the Nodes. From the networking perspective 802.11 5Ghz radios are connected with Ethernet over PCI-E interface and function as 802.11 WDS/Bridges (equivalent to L2/LAN Switch functionality). Since assumed that 802.11 5Ghz radios are always part of the Transport Domain (always the default domain in JNET2 system), therefore these radios are completely managed by the Networking Processor (meaning no IP representation required to the network). The internal communication for management purposes between radios &  is part of the Management Architecture (TBD – part of other SDR)

- 802.11 2.4 GHz AP – This host connected to the Network processor with Ethernet over PCI-E and gets its IP address from the assigned IP Domain (identified by SVLAN ID). This IP address is used for AP management client traffic forwarding, for one or more tunnels establishments to the Operator / ISP Networks (e.g. L2TP, GRE, …). 802.11 2.4 GHz AP should support the following networking functions (related to the JNET2 transport only)

  - SVLAN/IP/… termination and origination. It can be multiple SVLANs (e.g. per SSID) - the specific WIFi Access connectivity solution options specified in other SDR.

  - DHCP Client

  - UDP/TCP port forwarding for the appropriated functions (e.g. Port # <-> Management Client)

  - TCP/IP stack

  - Various and multiple tunnels types over IP (e.g. L2TP, GRE, …) – the specific WIFi Access connectivity solution options specified in other SDR.

  - 802.1p Class Of Service support (QoS) coloring

- Other IP-Hosts (Applications) connected to the Networking Processor over Ethernet (in case of external L2 switch (between the Networking Processor and multiple IP-hosts), it should support VLAN tagging / switching. Examples for IP-Hosts: JNET1 GateWay, IP-Cameras, ZigBee GW and etc. Actually, the networking functions per each IP-Host similar to those of 802.11 2.4Ghz AP.

NOTE: Single Domain Case is the private case of the Multi-Domain. In the Single Domain Case all Application Hosts Belong to the same IP Network Domain. The Single Domain Case is shown in Figure 14.

Figure 15: Single-Domain JNET2 Node Networking Communication Node Diagram

As can be seen from Figure 14 all Application Hosts (e.g. Sensors behind Application processor, 802.11 2.4GHz AP, other IP Devices) connected to the Network Processor and get the same IP address (IP10) as Network Processor represented to the infrastructure network. The internal traffic forwarding is based on the Juganu predefined UDP/TCP port’s numbers (as defined in Section ‎5.2.3). SVLAN0 is the default VLAN ID tagging.

Important NOTE: As part of the roadmap decision it is possible to support Single Domain Case as the first.

# Functional Requirement – F0002, Internal MAC Addressing structure

Table 1 shows the JNET2 Node MAC addressing requirements.

| JNET2 Node Block | MAC Type (See Note 1) | Requirement Description |
| --- | --- | --- |
| Network Processor | Globally Unique MAC address (Juganu OUI pull) | This MAC can be exposed to other non-Juganu systems. |
| Application Processor | Private MAC address – no uniqueness required | All Application processors are provisioned with the same MAC. MAC = AA AA AA AA |
| 802.11 5Ghz radios | Juganu Unique MAC address (Juganu CID pull) | One MAC per radio. This MAC address used for Ethernet communication with Network processor and for 802.11 address fields (per mode) |
| 802.11 2.4Ghz AP | Globally Unique MAC address (Juganu OUI pull) | This MAC or BSSID derived from the MAC (using same OUI) is exposed to WiFi users, therefore should be globally unique. (For OUI refer NOTE 2) |
| Other IP Hosts | Juganu Unique MAC address (Juganu CID pull) or any 3rd party |  |

Table 1: JNET2 Node MAC addressing

Notes:

- The IEEE defines “Guidelines for Use of EUI, OUI, and CID” which is relevant for different types of MAC addresses any network block should use. There are 3 types of MAC addresses defined for JNET2 system

  - Globally Unique MAC – IEEE defined OUI (Organization Unique Identifier – 24 bits). Using this OUI + additional 24bits extension proves that MAC address is globally unique. Since lack of MAC addresses the IEEE defined 3 classes for unique MAC prefix assignment. As can be seen from Table 2 Juganu should acquire MA-M address segment, which allows generating ~1 million globally unique MAC addresses.

  - Juganu (Company) Unique MAC – IEEE defined CID (Company ID – 24 bits). CID can be used as OUI (for example in 802.11 LLC header). CID does not prove global uniqueness, but since not public MAC collision probability is very low. As can be seen from Table 2 Juganu should acquire CID address segment, which enables ~16.7 million MAC addresses.

Table 2: IEEE OUI and CID assignment

  - Private (non-unique MAC) – same MAC in all JNET2 Nodes. Used only for the internal Node communication

- In case of MA-M (28 IEEE assigned bit range) the OUI can be used as 24 MSB (for example for 802.11LLC header). IEEE does not define OUI or CID for MA-M assignment as shown in Table 2, since it is not globally unique. However, the OUI collision probability is low.

# Functional Requirement – F0003, Internal IP Addressing structure

Table 3 shows the IP address structure within the JNET2 Node

| JNET2 Node Block | IP Address | Requirement Description |
| --- | --- | --- |
| Network Processor | - Single Outer IP address - Inner IP address (tbd) – 0.0.0.1 - POP/CAP IP address (tbd – optional) | - Outer IP address is used for communication with outside world (within and outside the Bubble). This address identifies the JNET2 transport host (e.g. Management Client) within the Transport IP-Domain (identified by SVLAN0). This IP is assigned by the DHCP Server belonging to the JNET2 Transport Operation Infrastructure (after the specific Node was connected to the Bubble Tree Topology and connectivity with the DHCP server is available. - Inner IP address (tbd) – can be used for intra-Node communication only – e.g. for local DHCP Server, intra-node communication with other blocks, local technician communication - Inner IP address – can be used for Direct Intra-Node communication (refer to Section ‎5.10 ) - POP/CAP IP address – as optional IP address assigned by the external transport IP network (e.g. transport over Internet) – this functionality is covered in the POP/CAP Node outbound connectivity requirement section in this SDR. |
| Application Processor | - Single Outer IP address per IP-Domain (identified by VLANs) - Inner IP address (tbd) – 0.0.0.2 | - As default (on SVLAN0) Application Processor is assigned by the same IP address as a Network Processor (it can be assigned via Juganu proprietary provisioning by the Network Processor or intra Node DHCP process with the Network Processor). In addition for multi-domain system Application Processor gets IP address per SVLAN via DHCP procedure from the DHCP Server locate in the relevant IP-Domain - Inner IP address (tbd) – can be used for intra-Node communication only – e.g. for local intra-node communication with Network Processor prior the networking topology establishment. |
| 802.11 5Ghz radios | - Inner IP address (tbd) – 0.0.0.2 | - Inner IP address(tbd) – same as Application Processor |
| 802.11 2.4Ghz AP | - Outer and Inner IP address – 0.0.0.2 | Same as Application Processor |
| Other IP Hosts | - Outer IP address | It is assumed that each external IP host will require single IP to be connected. If just SVLAN0 exists, the IP host will be assigned with the same default IP as the Networking Processor. In case of different SVLAN the DHCP process will be done against the DHCP Server in the predefined domain. (this requirement should be revised by case and depends on the 3rd party IP Host solution – e.g. IP Camera, …) |

Table 3: JNET2 Node IP addressing

# Functional Requirement – F0004, Internal UDP/TCP Port structure and communication principals

The Table 4 shows the UDP/TCP ports numbering as should be allocated per each host. The table defines Juganu ports # (non IANA official ports), these port numbers should be used by the appropriated Applications for Server to Client or peer-to-peer communication within the Bubble.

| JNET2 Node Block | UDP/TCP ports numbers | Requirement Description |
| --- | --- | --- |
| Network Processor | 7500 – 7509 | 7500 – Management Client 7501-7509 – reserved Note: can be assigned different ports per each management process (TBD – part of the Management Architecture) |
| Application Processor | 7510 - 7524 | 7510 – Transport Management Client 7511 – Camera 1 (refer to MIPI port 1) 7512 – Camera 2 (refer to MIPI port 2) 7513-7516 – reserved for cameras applications 7517-7524 – reserved for other sensor applications |
| 802.11 5Ghz radios | Not assigned |  |
| 802.11 2.4Ghz AP | 7530-7534 without port | 7530 – Transport Management Client 7531 – 7534 - reserved Non-port – for WiFi AP user traffic (e.g. L2TP, GRE tunneling) Any other ports based on the WiFI AP forwarding policy for the user traffic |
| Other IP Hosts | TBD | Should be decided per case |

Table 4: JNET2 Node Blocks UDP/TCP port numbering

The Port number allocation is based on the following rules:

- Ingress packet – Destination Port (e.g. from Server to Client)

- Egress Packet – Source Port (e.g. from Client to Server)

- Peer-to-Peer Packet – Destination and Source Ports (e.g. from Camera 1 in Application Processor of JNET2 Node A to Camera 2 in Application Processor of JNET2 Node - for this example: Source=7511 & Dest.=7512)

NOTE: this approach proves that any NAT on the Transport (outside of Juganu Bubble segment) will not impact the port numbering.

# Functional Requirement – F0005, JNET2 Node initial setup procedure

The Figure 15 shows the JNET2 Node initial setup process for multi-domain case.

Figure 16: JNET2 Node Networking Initialization Process for Multi IP-Domain

Detailed description of the Networking initialization process:

- The system powers up with the following default and factory parameters:

  - MAC addresses – as defined in Section ‎5.2.1

  - Inner IP addresses – as defined in Section ‎5.2.2

  - UDP/TCP Port Numbers – as defined in Section ‎5.2.3

- Using Inner IP addresses, the Network Processor (as a Master Block within the JNET2 Node) may communicate with other Blocks using Inner IP addresses and Port numbers – the messaging and internal procedures are out of scope of this document

- The JNET2 Node is associated to the upper hierarchy Node within the JNET2 Bubble and becomes one of the Tree Topology members. In this step the Outer VLAN ID is provisioned to the Node. (the Outer VLAN ID provisioning procedure is out of scope and specified in the JNET2 Topology Specification SDR). This step includes the Tree Forwarding Table provisioning (the table structure and provisioning procedure are defined in the next sections of this SDR)

- The JNET2 Node Network Processor provides DHCP procedure with DHCP Server over SVLAN=0 and gets IP address (IP10)

- The JNET2 Node Network Processor communicates with JNET2 Transport Management Server and configured with appropriate Transport Networking rules (related to the multi-domain SVLAN configuration – described in Section ‎6). As result of this configuration Network Processor may configure the associated hosts (e.g. Application Processor, WiFi AP) with the proper Transport Networking rules. (e.g. SVLAN ID ≠ 0 and IP10 if SVLAN=0).

NOTE: This step assumes that all JNET2 Nodes within the same Bubble have completely similar Network Transport rules, therefore the configuration can be propagated directly within the Bubble (e.g. from CAP/POP Node or any other neighboring Node) or from the Central Management Server (Out of scope of this document).

- In case of SVLAN ≠ 0 the Network Processor executes DHCP procedure with DHCP server located in the appropriate IP-Domain (identified by SVLAN ID) on behalf of the associated host and gets IP address. Network Processor will configure the IP address the associated host internally (Juganu inner Node configuration). Network Processor should support the following capabilities which are explained in details in the other sections in this document:

  - ARP table per SVLAN – refer to Section ‎5.4

  - ARP Proxy – replacing MAC addresses of other host by own MAC address – refer to Section ‎5.4

- Direct communication between the Application Host and Servers or Peer-to-Peer established. It is under the Application Layer responsibility to learn the Application instances (e.g. Application Server URL, other application peers)

Note: The Network Processor should have additional capabilities to support Private WiFi Access Service – LAN based (e.g. Police or Municipality Private network). Such capabilities include ARP Proxy / Dynamic ARP table management and DHCP sniffing – more details are in the WiFi Private Access Service SDR.

The Figure 16 shows the JNET2 Node initial setup process for the single domain case.

Important NOTE: As part of the roadmap decision it is possible to support Single Domain Case as the first.

Figure 17:JNET2 Node Networking Initialization Process for Single IP-Domain

The main difference between the multiple and single domain is lack of SVLAN policies provisioning and IP addressing per SVLAN DHCP procedures. In the Single Domain the single IP address provisioned for all hosts and the Network processors forwards the traffic internally using UDP/TCP Ports. Non UDP/TCP traffic (e.g. GRE, IPSec, L2TPoIP tunnel) will be always forwarded to and originated by WiFi AP 2.4Ghz (if on the same IP-domain).

# Functional Requirement – F0005, End-to-End downstream traffic forwarding principals

The Figure 18 shows the packet flow principal diagram from the network to the destination Application Host within the JNET2 Node.

The concept defines specific Juganu improvements to enable the traffic to flow through multi-hop tree topology (can be 20 and even more hops). This concept enables the following values:

- Minimum processing time in the transit nodes for e2e traffic – assumed that most of the traffic is e2e and the latency reduction is one of the major motivations

  - No ARP table lookup per VLAN in transit nodes

  - No SVLAN switching and/or remarking in the transit node

  - Small Outer VID forwarding tables within the transit node (become smaller for higher tree hierarchies)

  - Upstream traffic forwarding with no tables’ lookup

- Good coexistence with other intra-bubble traffic management requirements, like as IP-Multi Domain, peer-to-peer traffic, broadcast traffic – for these types of services, the Juganu traffic management solution reduces the processing within the transit nodes. For example in case of peer-to-peer traffic with the 20 hops’ tree the packet may transit through 40 hops.

Figure 18: Principal E2E downstream packet flow

The Table 5 determines the e2e downstream packet flow and reference sections in the document for the detailed requirement.

| Step # | Description | Reference Section/ Requirement |
| --- | --- | --- |
| 1 | The ingress packet enters the POP function within the GNET2 CAP/POP Node from the transport tunnel (examples for tunnel types are shown in the Figure 21). The POP function terminates the transport tunnel and associates its Tunnel ID with the SVLAN ID tag within the GNET2 Bubble. Based on the Dest. IP field with the packet the POP function determines the Dest. GNET2 MAC address (using the ARP table) and based on the MAC<->Outer VID mapping the POP function adds Outer VLAN tag. POP Function signs the traffic with E2E flag within the Outer VLAN tag. 802.1p is colored as well for either Outer and Inner VLAN tags. | Section ‎5.4.1 |
| 2 | The packet approaches the JNET2 Transport Function (within the POP/CAP Node). The packet header must include the following fields: - Dest. MAC address (MAC address of the Network Processor belonging to the destination JNET2 Node) - SVLAN ID - Outer VLAN ID - Type of Service (in both VLAN tags) – 802.1p based. In the initial stage both fields will be the same value. - Dest. IP address – destinated to the targeted host (e.g. Cameras served by Application Processor, 2.4Ghz WiFi AP) belonging to the destination JNET2 node within the Bubble. - Dest. UDP/TCP port# (or unnumbered) – used by the Destination Network Processor to forward the traffic to the appropriated Host (within the Node) | Section ‎5.4.2 |
| 3 | If Transport Function identifies that Outer VID is different from its own, then based on the Outer VID to AP<->STA link mapping table the packet is transmitted in DL direction over 802.11 5GHz radio (assuming that AP has multiple associated STAs). If Outer VID belongs to the Node, then the Node is the Destination Node and behaves as defined in the step 5. | Section ‎5.4.2 |
| 4 | When that packet approaches the Transit Node it behaves as defined in Step 3 |  |
| 5 | When the packet approaches the Destination Node (the Transport Function identifies Outer VID=own ID), then the Network Processor forward the packet based on Dest.IP, port # and SVLAN ID. Since the Destination Network Processor functions as Proxy ARP it should replace the dest. MAC address with the appropriate one per the destination host. | Section ‎5.4.3 |

Table 5: E2E Downstream Packet flow steps

# Requirement – F0006, POP/CAP Node E2E Downstream Packet Forwarding Requirement

The Figure 18 shows the downstream packet flow within JNET2 POP/CAN Node.

Figure 19: POP/CAP Node e2e downstream packet flow

As can be seen from Figure 19 the POP/CAP Node comprises two Functions:

- POP Function – responsible to intermediate between the external transport environment to the JNET2 Bubble transport. POP Function is resided only in the POP/CAP Node (NOTE: can be implemented internally or as Stand Alone Juganu GW Product – TBD)

- Transport Function – common function as in any JNET2 Node. For Downstream traffic the Transport Function operates in Transit or Termination modes (functionality explained in the Section ‎5.4.2)

POP Function must support the following mapping tables for DL Forwarding Tunnel:

- Table Table 6 presents the Tunnel Mapping to SVLAN ID example.

| External Tunnel ID | SVLAN ID | JNET2 Service Type | Functionality | Support Release |
| --- | --- | --- | --- | --- |
| Untagged or VLAN ID=0 | 0 | e2e | Used as default system configuration. Allows to connect the POP/CAN Node to the Management System during the initialization | 1 - Mandatory |
| VLAN ID = 105 | 105 (or other) | e2e | Used for Multi-domain. Provisioned in the system | TBD – can be mandatory for WiFi Access Service (e.g. per SSID) – not part of the scope in this SDR |
| L2TPv3 tunnel id AA | SVLAN 210 (or other | e2e | L2TPv3 tunnel ID can be identified by the IP end point and/or Session ID (no specific requirement in this SDR) | TBD – may be release 1 |
| MPLS PE Labeling | SVLAN 305 | Peer to peer | MPLS has various option. In any case it is assume only L2VPN MPLS mode support or pseudo wired. (should be defined with the first customer). The easiest way to connect to MPLS is functioning as CE. In case of CE the requirement is similar to VLAN ID support. | Future release |
| Other tunnel types |  |  |  | Future release |

Table 6: External Tunnel mapping to SVLAN

There are various options for external tunneling support. As initial requirement for this SDR the system should support:

- Untagged traffic – VLAN = 0 or no VLAN tagging. This is the default system functionality. The external transport should use this type of “tunnel ID” in order to connect Juganu JNET2 Bubble to the Transport Management Domain. (Note: it can be the requirement that other external tunnel types will be used for Transport Management Domain. This requirement is not supported at the beginning and should be specified once raised by the customer.)

- Other external tunnel ID types are used for multi-domain system (each external connectivity option which is not basic will be added as new Requirement ID in the SDR):

  - External VLAN ID (≠0) – must be supported by the system and mapped to the SVLAN ID (can be the same value in most cases, if no overlapping)

  - L2TPv3 tunnel – TBD. Since it is the first approach implemented in Juganu need to consider supporting it (not part of this Requirement)

  - MPLS L2 tunneling – TBD (not part of this Requirement)

  - Others – TBD

The JNET2 Service Type parameter in Table 6 refers to Juganu PSTI sub-field as shown in Figure 13. The Service Type may be provisioned as E2E or Peer-to-Peer. DL and UL PSTI refers to the traffic direction for E2E traffic and Broadcast refers to the L2/Ethernet broadcast packets (Dest MAC=FF:FF:FF…). As can be understood the Service Type parameter and PSTI are not directly associated. If Service Type Parameter = e2e, the PSTI=DL e2e (for downstream unicast packets). DL traffic can be flagged as PSTI=DL e2e or Broadcast only.

Important requirement clarification!!! The SVLAN ID and JNET2 Service Type mapping should be provisioned with the same values in all JNET2 Nodes within the Bubble. This is part of the Bubble Topology profile (more details in the Topology SDR)

- Table 7 presents the ARP Proxy table example mapping Destination IP address to the Destination JNET Node MAC and Outer VLAN ID (as shown in Figure 19).

NOTE: in the current implementation the JNET2 NODE MAC (Network Processor) and Outer VLAN ID are 1:1 associated, but this separation in future can be used for various capabilities where more flexible mappings can be required (TBD).

| SVLAN ID | Destination IP Address | Destination Node MAC | Outer VLAN ID | Host Type Indicator | Timer |
| --- | --- | --- | --- | --- | --- |
| 0 (default) | 10.10.10.10 | A1 | 105 | 0 - Internal | Long (static) |
| 0 (default) | 10.10.10.11 | A2 | 106 | 0 - Internal | Long (Static) |
| 105 | 5.5.5.10 | A1 | 105 | 0 - Internal | Long (Static) |
| 105 | 5.5.5.11 | A2 | 106 | 0 - Internal | Long (Static) |
| 220 | 192.167.5.2 | A1 | 105 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.3 | A2 | 106 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.4 | A2 | 106 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.5 | A3 | 106 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.6 | A1 | 105 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.7 | A1 | 105 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.8 | A3 | 107 | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Short (Dynamic) |
| 220 | 192.167.5.1 | Subnet IP Router for SVLAN=220 (learned from the DHCP process as per Section ‎5.8) | Only for upstream traffic | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Long |
| 220 | 255.255.255.0 | Subnet Mask for SVLAN=220 (learned from the DHCP process as per Section ‎5.8) | In downstream – discard all traffic which not belong to the subnet In upstream – forwarding toward the Subnet IP Router | 1 – WiFi LAN Access or other LAN transport service (used for peer to peer) | Long |

Table 7: POP Function Internal JNET2 Bubble ARP Table

Table 7 shows two mapping types as indicated by the Host Type Indicator:

- 0 (Internal) – identifies the tuples, which represent internal hosts within the Node (e.g. Cameras, WiFi AP, …). As per proposed implementation such hosts’ Destination IP are assigned to the Network Processor (in the related JNET2 Node) per SVLAN.

- 1 (WiFi LAN Access) – referred to the WiFi Access users connected as LAN users (e.g. private network: Police, Muni Workers). This SDR does not define this feature, but as part of the future-looking architecture this service can be supported as part of FOAM.

The ARP Table has two timers’ types to manage the table dynamically:

- All internal hosts are typically static and therefore no need refresh the ARP table frequently. Therefore, ARP table update can be done by various techniques (will be decided during the implementation):

  - Standard way - DHCP sniffing (as specified in Section ‎5.8), packets sniffing from the JNET2 host nodes, ARP procedures. In this way if POP Function receives packet with uknown destination IP address (not in the table) it should send broadcast ARP to all Nodes in the Bubble to learn the Dest. MAC. As part of the DHCP sniffing the POP function should store the Sub-Net per SVLAN ID in order to filter the fault packets with Dest. IP address not belonging to the IP Subnet.

  - Juganu preconfigured approach – once the JNET2 Node joins the Tree Topology and IP address is accepted (via DHCP or Static/Preconfigured), the Node will update the POP Function with its IP address and MAC address (the update mechanism can be defined as part of the Topology SDR or manually preconfigured – in the initial stage)

- Short (Dynamic) – these tuples manage the ARP table for WiFi LAN users (relevant for private network only) and therefore the update time should be short and table update should be done by standard procedures (as explained above)

[IF NEEDED THE ARP TABLE UPDATE PROCEDURES CAN BE ADDED TO THIS SDR]

- As part of the Proxy ARP functionality the POP Function will acknowledge all ARPs Requests from the outside transport domain per VLAN, which destinated to the IP addresses as per Table 7. In addition POP Function Proxy ARP should learn the Gateway Router MAC address per SVLAN (assumed one GW Router per SVLAN – this router IP is presented in the DHCP Option 3 and explained in Section ‎5.8). As result the system should manage the Table 8 and dynamically update it (with predefined timer). Based on this table the POP Function will hide the Transport MAC addresses from the JNET2 Nodes hosts (by replacing it with the POP/CAP Node MAC address).

| SVLAN | Transport/GW Router IP | Transport Router MAC |
| --- | --- | --- |
| 0 (default) | 10.10.10.1 | BA: …. |
| 210 | 5.5.5.1 | 1F: … |

Table 8: POP Function Transport Proxy ARP Table

# Requirement – F0007, Transit Node E2E Downstream Packet Forwarding Requirement

Figure 20 shows the packet flow Transport Function requirements in the Transit Node for DL e2e traffic.

Figure 20: Downstream e2e Packet Flow in Transit Node

The 802.11 packet approaches the Transit Node from the higher hierarchy JNET2 Node AP (Parent). The ingress 802.11 MAC header comprises four MAC addresses and VLAN Tags (as shown in Figure 20). The Transport Function checks the Outer VLAN ID and PSTI. If PSTI=DL e2e the Transport Function forwards the Packet to the next hop based on the Table 9.

| Outer VLAN ID | Link # / 802.11 Addr1 (Rec. Add) | Description |
| --- | --- | --- |
| 112 | my | The packet belongs to the Node (traffic Termination Node – functionality explained in Section ‎5.4.3) |
| FF… | Broadcast: packet is broadcasted to all STAs | PSTI=”broadcast” (refer to Section ‎5.7) |
| 120 | Link #1/Addr1=A2 (BSSID STA1) | PSTI= DL e2e |
| 121 | Link #2/Addr1=A3 (BSSID STA2) |  |
| 122 | Link #2/Addr1=A3 (BSSID STA2) |  |
| 123 | Link #1/Addr1=A2 (BSSID STA1) |  |
| 125 | No entry | PSTI = DL e2e, fault situation (the forwarding tables in the JNET2 Tree are not consistent) |

Table 9: JNET2 Node Transport Function DL forwarding table

# Requirement – F0008, Termination Node E2E Downstream Packet Forwarding Requirement

As explained in Section ‎5.4.3 when the packet approaches the JNET2 Node Transport Function and identifies the matching between the Outer VLAN ID and own ID, it behaves as a packet Termination Node. Figure 21 shows the Termination Node packet flow diagram.

Figure 21: Termination Node e2e downstream packet flow

Termination Forwarding Table example is shown (see Table 10)

| SVLAN | Dest IP | Dest Port | Dest MAC (Host) | Description |
| --- | --- | --- | --- | --- |
| 0 (default) | 10.10.10.10 | 7500 | Internal host | Transport Management (Network Processor) |
| 0 | 10.10.10.10 | 7511 | MAC6 | Camera 1 host (Application Processor) |
| 0 | 10.10.10.10 | non | MAC2 | 2.4 GHz WIFi AP |
| 0 | 10.10.10.10 | 7530 | MAC2 | WIFi AP / Transport Management |
| 210 | 5.5.5.11 | 7514 | MAC6 | Sensor 1 host (Application Processor) |
| 210 | 5.5.5.11 | 7515 | MAC6 | Sensor 2 host (Application Processor) |

Table 10: Termination Forwarding Table

Table 10 does not include the WiFi AP users (this functionality is defined in the WiFi AP Access SDR). The table has Dest. IP address/Port # to Dest MAC address mapping, which refers to the Proxy ARP functionality supported by the Network Processor in any Termination Node. The Proxy ARP functionality should terminate all ARPs and practically hides the IP/MAC hosts behind each JNET2 Node Network Processor. Thus reducing/eliminating the ARPs in the system. For e2e traffic support the JNET2 Node should update the POP Function in the POP/CAP node with own ARP table (Dest. IP -> Dest MAC) (as explained above, this update in POP Function ARP table can be done by DHCP / upstream packet sniffing as clarified in Section ‎5.8 or Juganu proprietary procedure).

For the Internal JNET2 Node Hosts the mapping between the Port # & Host MAC is common between all Nodes (default value in SW). The SVLAN mapping to the Port # is common between all Nodes within the Bubble.

The Juganu Port numbers and MAC addresses for the internal JNET2 Nodes communication are defined in Sections ‎5.2.1 and ‎5.2.3.

# Functional Requirement – F0009, End-to-End upstream traffic forwarding requirement

The Figure 22 shows the upstream traffic flow from the Originating Node through Transit Nodes toward POP Node and forwarding to the outside transport segment.

Figure 22: E2E Upstream packet flow in the JNET2 Bubble

In the tree topology the Upstream e2e packet flow is the simple and the required functionality per each Node type is explained in the Table 11.

For E2E Upsteam traffic the Service Type for the specific SVLAN should be provisioned as “E2E” service.

Important Note! Any packet originated in the JNET2 Node for the E2E service should get out of the Bubble through the POP/CAP Node (never terminated by any other Node within the Bubble)

| Node Role | Functional Steps Requirement |
| --- | --- |
| Originating Node | 1. The packet received by the Transport Function from the Application Host contents the following header: Source MAC = Host MAC Dest MAC = Network Processor MAC (based on Proxy ARP function as explained in Section ‎5.4.3). MAC can be FF:… (broadcast, e.g. for DHCP) SVLAN tagging = as pre-provisioned Source IP Address = Host IP Dest IP Address = Gateway/Transport Router IP (per Domain) or any IP not belonging to the Sub-NET (relevant for Private WiFi LAN Access service or other peer-to-peer LAN service – similar Table 7) Source Port # = Host Port or NO Port (as per Table 4) Dest Port# = any 2. Transport Function generates the 802.11 packet and sends it in upstream direction setting the following values in the header: Source MAC = Network Process MAC (replaced) Dest MAC = POP/CAP Network Processor MAC (replaced) or FF:.. (broadcast) not replaced SVLAN tagging = same Outer VLAN tagging (VID + Type Of Service field)– new PSTI = UL e2e Node ID (10 bits) = OWN ID (the motivation to indicate the POP/CAP Node easily the Node origin of the packet) TOS = copy from SVLAN TOS Other fields (IP, Ports) – no change 3. The packet is sent from STA (Child) to upper hierarchy AP (Parent) |
| Transit Node | The transport function checks the PSTI in Outer VLAN tag. If PSTI=UL e2e, it delivers the packet ASIS via 802.11 link (STA->AP). |
| CAP/POP Node Transport Function | 1. The upstream packet approaches the POP Transport Function and PSTI is checked. 2. If PSTI=UL e2e the packet is delivered to POP Function |
| CAP/POP Node POP Function | 1. POP Function removes the Outer ID header 2. The POP Function generates the new Packet to be sent to the Outside Transport segment with the following header Source MAC = CAP/POP Network Processor MAC (replaced) Dest MAC = GateWay/Transport Router MAC (replaced) (based on Proxy ARP function as per Table 8). Dest MAC can be FF:… (broadcast, e.g. for DHCP) and not replaced Tunnel ID (different types) – 1:1 mapping with SVLAN (as explained in Section ‎5.4.1 and Table 6) Other fields (IP, Ports) – no change 3. Packet is sent outside |

Table 11: Upstream e2e packet flow functional steps

# Functional Requirement – F0010, Peer-to-peer traffic forwarding requirement

[NOTE: This functionality can be implemented in the future releases]

The Peer-to-Peer functionality enables each host within the bubble to send the packet to each other internally (without getting out of the Bubble). Such functionality is enabled once the hosts belong to the same IP-Domain (i.e. SLVAN). The feature does not allow peer-to-peer packet forwarding within the Bubble between the IP-Domains.

In order to differentiate between e2e SVLAN and peer-to-peer SVLAN, the SVLAN should be provisioned (in all Nodes) as “Peer-to-Peer” Service.

When the packet comes from the outside transport and mapped by POP Function to SVLAN, which is provisioned as “Peer-to-Peer”, it is managed as defined for e2e downstream flow (refer to Section ‎5.4).

The special functional requirement for Peer-to-Peer traffic refers to the packet originated by one of the JNET2 Nodes (“Originating Node”) and the relevant junction Transit Node Transport Function, which should recognize the packet destination Node (per Outer VID). The principal peer-to-peer packet flow is shown in Figure 23.

Figure 23: Peer-to-Peer traffic flow diagram within the Bubble

The Table 12 explains the peer-to-peer packet flow process per the Node role as shown in the diagram above.

| Node Role | Functional Steps Requirement |
| --- | --- |
| Originating Node | 1. In order to support peer-to-peer traffic all JNET2 Nodes in the Bubble should have the mapping table (ARP Table for all Nodes per relevant SVLAN) as exists in POP Function (refer to Table 7). NOTE 1 2. If SVLAN Service Type = “peer-to-peer”, the Outer VLAN ID is set to Termination Node ID (i.e. 109 as per figure above) as per Table 7 Dest IP. to Dest MAC to Outer VID mapping 3. If the Outer VID matches any value as defined in Table 9 the Node should function as “Junction Node” for peer-to-peer traffic. Otherwise it sends the packet in upstream direction with following header fields: Source MAC = Network Process MAC (replaced) NOTE 2 Dest MAC = Term. Node Network Processor MAC (replaced) – i.e. 1C as per figure above NOTE2 SVLAN tagging = same Outer VLAN tagging (VID + Type Of Service field)– new PSTI = Peer-to-Peer Node ID (10 bits) = Term. Node – i.e. 109 as per figure above TOS = copy from SVLAN TOS Other fields (IP, Ports) – no change |
| UP Transit Node | 1. Check PSTI, if PSTI = Peer-to-Peer 2. Then check Outer VID value defined in Table 9. If Outer VID matches the value in the table then the Node should function as “Junction Node” for peer-to-peer traffic. 3. If not, it functions as UP Transit Node and should deliver the packet upstream ASIS. |
| Junction Node | Same as Steps 1 & 2 for UP Transit Node. If the Outer VID matches the value in Table 9, then it forwards the packet downstream according to the Table 9 and changes PSTI value to DL e2e (all other header fields are not changed). |
| DL Transit Node | Behaves as specified in Section ‎5.4.2 |
| Termination Node | Behaves as specified in Section ‎5.4.3 |

Table 12: Peer-to-Peer packet flow functional steps

Notes (for the table):

- The Table 7 update procedure from POP Function to all Transport Functions in all Bubble Nodes is out of scope of this SDR (should be defined as part of the Internal Management Procedures – SDR TBD)

- The Proxy ARP should be supported as explained for Originating Node in Table 11 and as per Section ‎5.4.3  .

# Requirement – F00011, Broadcast Traffic Requirements

NOTE: The Transport Network architecture considers that ARP and DHCP Broadcast packets (Dest MAC = “all 1s”) are not distributed as broadcast packets within the JNET2 Transport Network. Therefore, broadcasting support can be and will be avoided in Release 1. There is the difference between P2 and P3 systems:

- P2 – using OMNI antenna the LAN broadcasting is naturally supported by 802.11, however it wastes capacity as transmitted with the lowest rate.

- P3 – sectorized antenna. In order to support broadcasting the AP should send to all STAs as duplicated unicast traffic (refer to the Scheduler SDR004). Since this capability will not be supported in the Release 1, therefore no Broadcasting will be done.

In the Release 1 the PSTI will not be set to “Broadcast” value. The meaning of this requirement for the Release 1:

- ARP packets are terminated locally within the Nodes as part of the Proxy ARP functionality and ARP tables within the Bubble are updated by ARP Proxy mechanisms, which do not require any ARP broadcasting within the Bubble (as explained in this document)

- DHCP procedure is explained in Section ‎5.8 .

- All other (unrecognized) broadcast packets will be handled as following:

  - Packets from the external transport segment (terminated by the POP Function) – should be dropped by the POP Function if unknown. In case Juganu will have the Application, which will use the Broadcasting as part of the application functionality (e.g. server sends broadcast message to all sensors), the specific of this support should be defined.

  - Packets in upstream direction originate in one of the Nodes will be handled as Upstream e2e traffic.

For Future Releases: The Transport architecture considers PSTI=”Broadcast” in Outer VLAN ID field. In this case the Outer VID will be set to the Original Node ID (e.g. POP/CAP Node ID) in order to recognize the Broadcaster Origin. The Broadcast packet distribution will be allowed only for SVLAN Service Type = Peer-to-Peer.

# Requirement – F00012, DHCP Support Requirements

As shown in Figure 24 the DHCP Server resides in the Network / Operator Domain and DHCP Client is one of the Hosts (e.g. Application Host) located in one of the JNET2 Nodes within the Bubble. Therefore, as a general solution even the DHCP Procedure uses Broadcast MAC address the Bubble will manage the DHCP messaging flow as a unicast one between the DHCP Server and DHCP Client.

The Figure 24 shows the DHCP Procedure delivered through the JNET2 Bubble Functional Entities (Transport and POP Functions). In most of cases within the JNET2 Node the DHCP Client resides in the Network Processor (where the Transport Function located). In this case it is not required to support any DHCP Relay within the JNET2 Node. The DHCP Relay functionality is required when the IP Host is externally connected to the Network Processor (e.g. IP Camera or WIFi AP users in private network / LAN users – not in the scope on this SDR).

Figure 24: Initial DHCP Procedure example originated by the JNET2/Foam IP Host

The Table 13 explains the required DHCP functionality for Transport Function and POP Function which also are associated with the Traffic Forwarding and ARP Table update (using DHCP sniffing).

NOTE: In Release 1 it can be assumed that no DHCP Relay support required for the Transport Function (Host Client Side) as all Application Hosts are represented by the Network Processor (as per Figure 14).

| DHCP Message | Transport Function (DHCP Relay) | POP Function (DHCP Relay) |
| --- | --- | --- |
| General Requirements | 1. DHCP Function recognize Destination MAC address as “FF:FF:..” (broadcast) 2. Then DHCP Relay should check the UDP Port numbers. If 67 / 68 (per direction of the DHCP message) then it is DHCP Message 3. For all DHCP messages the DHCP Relay should identify DHCP Message Type (for the sniffing purposes and ARP table update as explained in this table) 4. DHCP Relay must Relay all message and does not manage any State Machine for the DHCP process |  |
| DHCP Discover -----------------> | 1. Replaces the Source MAC Addrs (Eth. Header – i.e. m1) with the Network Processor MAC Address (ARP Proxy function) 2. Client MAC within DHCP Message (i.e. m1) is kept and associated with the Network Processor MAC (i.e. MAC1). This association is a new entry for the Table 10 (the temporary IP address within the table is 0.0.0.0). 3. The Message is relayed ASIS on the relevant SVLAN and upstream unicast packet (PSTI=UL e2e) – NOTE: SVLAN ID should be sent by the host or identified by the Network Processor as a dedicated physical port | 1. Replaces the Source MAC Addrs (Eth. Header) with the POP Network Processor MAC Address – i.e. MAC10 (ARP Proxy function) 2. “Client MAC” within DHCP Message (i.e. m1) is kept and associated with the Network Processor MAC (i.e. MAC1). This association is a new entry for Table 7 (the temporary IP address within the table is 0.0.0.0 and temporary host MAC address, which will be deleted once a real IP address is learned). 3. The Message is relayed ASIS to the relevant Transport Tunnel ID (as per Table 6 ) |
| DHCP Offer <----------------- | 1. Replaces the Source MAC Addrs (Eth. Header – i.e. MAC10) with the Network Processor MAC Address – i.e. MAC1 (ARP Proxy function) 2. The message is relayed based on the Client MAC within DHCP Message (i.e. m1) to the relevant Host Destination based on the new entry in Table 10. (including SVLAN value) | 1. Replaces the Source MAC Addrs (Eth. Header – i.e. m2) with the POP Network Processor MAC Address – i.e. MAC10 (ARP Proxy function) 2. The Message is relayed ASIS on the relevant SVLAN as downstream e2e packet (PSTI=DL e2e). Tunnel ID to SVLAN mapping as per Table 6. |
| DHCP Request ------------------> | Similar to the DHCP Discovery message relaying steps (excluding Step 2, where no need to create new Table entry – if exists) | Similar to the DHCP Discovery message relaying steps (excluding Step 2, where no need to create new Table entry – if exists) |
| DHCP Response <------------------ | 1. Step 1 as DHCP Discover relaying 2. Replaces the temporary IP address in Table 10 (0.0.0.0) with “your IP” address within the DHCP payload – i.e. 1.1.1.10 (as result in the Table the IP address should be associated with the MAC Address as appears in “Client MAC” field within the DHCP payload (i.e. m1) 3. same as Step 2 in DHCP Offer relaying 4. (Optional – Note 1) Transport Function should use the “Subnet Mask” and “Router IP” within the DHCP payload in order to support LAN peer-to-peer service and update Table 10 (similar to the LAN peer-to-peer tuples as shown in Table 7) | 1. Step 1 as DHCP Discover relaying 2. Replaces the temporary IP address in Table 7 (0.0.0.0) with “your IP” address field within the DHCP payload – i.e. 1.1.1.10. As result in the Table the IP address should be associated with the Network Processor MAC (i.e. MAC1) in the tuple where the “Client MAC” (i.e. m1) field was learned (during DHCP Discovery process). The Client MAC value can be deleted (not used for the regular traffic forwarding) 3. Step 3 as DHCP Discover relaying 4. (Optional – NOTE1) POP Transport Function should use the “Subnet Mask” and “Router IP” within the DHCP payload in order to support LAN peer-to-peer service and update Table 7 |

Table 13: DHCP Procedure Support Requirements within the Bubble

Note:

- This capability is only relevant for peer-to-peer LAN transport service and may require additional system requirements (will be updated in the next versions)

The Figure 25 provides better clarification how the ARP / Forwarding tables (Table 7 and Table 10) are updated in Transport and POP Functions using the DHCP sniffing process.

Figure 25: Forwarding Tables update with DHCP sniffing

Other DHCP Requirements:

DHCP Client may initiate DHCP messages, which are unicast packets between the DHCP Client and the Server (e.g. DHCP Renew and DHCP Release). These packets are forwarded as any e2e unicast traffic (down&up stream).

No special treatment should be done for these unicast DHCP messages (in the initial Releases). It may create the mismatch between the Forwarding / Proxy ARP tables and the real host existence (till the timer will expire).  In case the Host will be rejected with the IP Renewal or no response from the DHCP Server, the Host will start the Broadcast DHCP process and then the relevant tuple in the Tables should be updated as per procedure explained above in this Section.

# Requirement – F00013, 802.1p TOS field support requirement

The 802.1p TOS (Type Of Service) field is part of the VLAN tagging 802.1Q header (refer to figure Figure 26)

Figure 26: TOS/Priority Field within 802.1Q VLAN header

802.1p field is used for the JNET2 Transport QOS support. The TOS is used to forward the packets into the appropriated priority queues. The QOS and queuing support is out of scope of this SDR, but this Section provides the basic requirements for the TOS field support.

- Since the JNET2 transport system uses double VLAN tagging (QinQ), it has TOS field in each one of them. In Release 1 Outer VLAN and SVLAN TOS fields should be the same value. All Transit Nodes will use Outer VLAN TOS field for QOS traffic priority (once it will be supported) – this will enable in future to make special QOS tricks within the Bubble and keeping end-to-end TOS value transparently using SVLAN TOS field.

- For ingress traffic (from outside transport segment) the POP function should sign the TOS value in SVLAN TAG based on the policy (which should be defined in QOS SDR Document). If the outside transport sends the packet colored with 802.1p tag, in this case the POP function should copy this value to the SVLAN TAG. The Default Value is TOS=1

- For egress traffic from the JNET2 Hosts the Default Value (TOS=1) will be signed by the Transport Function (prior to the specific QOS specification/SDR exists). The POP function should allow the SVLAN TOS field mapping to the appropriate QOS marking per the tunnel type (will be defined per the tunnel type support case). In case of VLAN tunneling the same TOS value will be copied by POP function from the SVLAN TOS field.

NOTE: This section defines very basic requirement for TOS/802.1p marking to allow basic service and right infrastructure at the Release 1. Additional TOS marking rules and transport QOS support is defined in other SDR. In any case for CSMA schedule the QOS support is very limited (specifically for upstream traffic). The TDMA system has better control to manage the transport QOS. In any case both CSMA and TDMA scheduled systems will be able to support multiple queues to enable traffic priority.

# Requirement – F00013, Direct inter-node communication

NOTE: This requirement only relevant for P3 system and later.

Direct inter-node communication refers to the capability, where two JNET2 Node may communicate directly independent to the specific topology or Bubble. This requirement comes to serve the following scenarios:

- Initial communication with other neighbors during the JNET2 Node power up, or bubble structure change, or beam/sector training process and etc. This communication link can be used to identify the neighboring Nodes, learning the environment (e.g. radio link quality, IDs, …), exchanging information for tree topology establishment/optimization/path redundancy and other topology establishment functions.

- Enabling specific ad-hoc node communication for different future services (not related to the general service transport traffic management as defined in this SDR)

The direct communication establishment steps

- Radio association between AP and STA (the process is part of the Scheduling SDR004).

- Authentication Process (refer to Section ‎5.12).

- IP communication establishment using DHCP Procedure, while AP side is used as DHCP Server side and STA is used as DHCP Client side. DHCP address allocation is based on the following principals:

  - As per Table 3 (Inner IP Address) the DHCP Server, Router IP is 0.0.0.1

  - IP Subnet is 255.255.255.0

  - IP allocation address block for neighboring Node DHCP Clients (STA side) can be any in range of 0.0.0.10 – 0.0.0.254

The 802.11 direct communication protocol stack is shown in Figure 27.

Figure 27: Direct Communication Protocol Stack

The 802.11 MAC mode is TBD (Router or WDS/Bridge).

# Requirement – F00014, JNET2 Technician Direct Communication Requirement

This requirement scope only related to the communication aspects. This capability should allow the authorized technician Terminal to connect to the lamp node directly when the technician is in the vicinity to the Node.

There two options for the Technician Terminal to connect to the lamp Node: via Bluetooth or 2.4Ghz WiFi (not decided yet). In both cases the communication should be allowed using TCP/IP stack.

Generic Requirements:

- Initial association: Since the technician may listen multiple lamp Nodes in the area, it required that the Node will advertise itself with unique ID. One of the options for WiFi AP is using Hidden SSID equal to the Node Serial Number (which is also known to the Technician) and BSSID can be derived from the SSID numeric value (to assure no collision with other neighboring lamp Nodes).

- Security / Authentication: It is required mutual trust authentication process. The proposed solution is using x509 certificate signed by the private key on both sides and local authentication TLS method (same method as used for the initial authentication between the nodes). In this case the technician computer is used as Client and the Node is used as Server. Refer to Section ‎5.12.

- IP -assignment (using DHCP). As for Direct Inter-Communication (see Section ‎5.10) the Node can assign the IP address from the Inner-IP address space as per Table 3 (e.g. Inner IP = 0.0.0.5, Router IP = 0.0.0.1, IP Subnet = 255.255.255.0).

- The Technician Terminal Management Client may use TCP or UDP Port (e.g. 7500) as defined in Table 4.

NOTE: The security requirement in Step 2 is relevant to secure the Technician Terminal communication with the selected lamp. It is possible that the Management Application will provide additional User authentication to allow the specific User priority access.

# F00015 - JNET2 Security X.509 / TLS certificate-based support requirements

The JNET2 system should support the X.509 certificate using TLS mutual authentication method. This security framework is used for three use cases:

- Inter-Node transport communication – as explained in Sections ‎5.10. Any communication between 5GHz AP and 5Ghz STA between the Nodes (as part of the Bubble or as Direct) should be secured

- Node authorization with the central database. The Database Server should periodically propagate the Node Black List (e.g. stolen nodes) to all Nodes in order to unregister the “bad” Nodes.

- Local Technician access via 2.4 GHz AP (independently whether Node is connected to the Bubble or not)

The Figure 28 provides better clarity for three scenarios, where the TLS/X.509 framework should be used.

Figure 28: JNET2 TLS/X509 Security Scenarios

Initial Requirements (Release 1)

- Juganu should establish (or sign with) the Certification Authority (CA) in order to generate the X.509 Certificates and Key Pair (Private and Public). In the initial stage Juganu may use OpenSSL in order to generate the Certificates and the Keys. Juganu should choose the traffic and signature encryption algorithm.

- Juganu should develop the procedure to provision own Certificate & Keys into each JNET Node and the Technician Terminal (should be specific computer provisioned with the Juganu Certificate) in the production

- In the authentication procedure the AP functions as a Server and the STA functions as a Client. Since both sides have pre-provisioned certificate the mutual TLS authentication method is used.

- After the certificates verification stage both sides should derive the Master Key (MK) and other keys (e.g. PTK, GTK) to be used for the 802.11 traffic encryption. The system uses 802.1x security model to enable EAP/TLS procedure as shown in Figure 29.

Figure 29: EAP/TLS Procedure for 802.11

- The system (SW & HW) should ensure that the Private Key is protected (nobody has access to read it). The protection level can be improved in the future. As initial requirement the provisioned Private Key cannot be read by anyone via SW.

- X.509 Certificate should be bind to the HW (MAC addresses) in order to be sure that Certificates are not duplicated

The X.509 template certificate is shown is Figure 30 and specific JNET2 Node & Technician Terminal fields within the Certificate are explained in Table 14.

Figure 30: X.509 Certificate Format

| X.509 Field | JNET2 Specific Values to be provisioned in the certificate |
| --- | --- |
| Version | 3 (the latest) |
| Serial Number | Serial Number of the JNET2 Node. Unique number registered in the database and used for the authorization To be used also for the Technician Terminal / Computer |
| Signature algorithm (for CA) | TBD (to be chosen from well-known) – not sure required if Juganu is the CA (???) |
| Issuer name | At the beginning: Juganu name (in future possibly authorized CA) |
| Validity period | Start Date: Production Date Expire Date: TBD (e.g. 5 years) – should be decided per the company policy. In more progressed releases the certificates can be uploaded / renewed with short periods for the different security / business usage options. |
| Subject Name / Optional: Subject Unique Identifier, Extensions | Need to provision 4 MAC Addresses: Network Processor MAC, 5GHz WiFi MAC x2 and 2.4GHz WiFi MAC. Network Processor MAC – used for the binding between the HW and the Certificate (during the production process) WiFi MACs – can be used in the future for binding between the WiFi BSSIDs with the certificate (not for Release 1) For the Technician Terminal can be used FQDN (e.g. for the Application) – need to decide the proper approach - TBD. |
| Subject Public Key | Public Key generated by the CA (e.g. OpenSSL) together with the Certificate |

Table 14: X.509 Certificate Fields Provisioning Requirements for JNET2 Node and Technician Terminal

(VERY IMPORTANT TO SPECIFY PER EACH FUNTIONAL REQUIREMENT THE IMMIDIATE REQUIREMENT FOR COMING SW RELEASE, BUT ALSO THE EXPECTED FUTURE IMPROVEMENT REQUIREMENT. IT IS IMPORTANT AS PART OF THE ROADMAP MANAGEMENT)

# Management Requirements

[To specify relevant requirements for the feature related to the new Configuration, Faults, Alarms, Events, Key Performance Indicators (KPI) / Counters in each sub-section]

[As intro to explain general Management concept requirements related to the feature if relevant

# M00001 - JNET2 Management Architecture - Networking Structure Requirement

This Section describes the networking structure related to the JNET2 Management architecture but does not cover complete Management Architecture. Actually this Section defines the connectivity infrastructure between different modules within the JNET2 Node and between the Nodes. It does not define topics like as Server/Client relations.

The Figure 31 shows the JNET2 Node networking structure used by the Management entities for the single domain and multi-domain cases.

Figure 31: JNET2 Node Management Connectivity Diagram

As can be seen from the Figure 31 the main difference between the Single and Multi-Domain that communication with the different mng entities happens via single or multiple SVLANs. In case of Single Domain the Forwarding Matrix within the Network Processor forwards the packets to the appropriated destination based on the predefined Juganu Port Numbers (as per Table 4), and in case of Multi-Domain the same Network Processor forwards the traffic using the combination of Dest. IP address and the Port Number. Using this connectivity approach the Management Client/Server may use standard TCP/IP stack between all Management Entities within the Node, with the Management Server in the CO (Central Office) and between the Nodes.

# Configuration Requirements

[To fill the below tables and if required to provide more detailed description for the new Configuration Parameters]

| Configuration Parameter Name | Short Description | Units | Value Range | Default Value | Notes and Reference |
| --- | --- | --- | --- | --- | --- |

| Configuration Parameter Name | Exposed to the User (YES/NO) | For Juganu Use only (YES/NO) | Notes for special use |
| --- | --- | --- | --- |

# Fault, Alarm, Event Requirements

[To fill the below tables and if required to provide more detailed description for the new Fault, Alarm, Events Parameters]

[If needed to provide detailed description of the specific functionality, which creates the Fault/Alarm/Event]

[Description: Each Event may have fault or alarm status. Fault means that system provides report and the description of the Fault Event. Some Faults may result to Alarms with different priorities

Alarm means the indication to the user on some event, which should be treated.

| Event Name | Fault / Alarm | Description | Alarm Priorities | Notes and Reference |
| --- | --- | --- | --- | --- |
|  |  |  | Critical |  |
|  |  |  | Major |  |
|  |  |  | Minor |  |

# Counters / KPI requirements

[To fill the below tables and if required to provide more detailed description for the new Counters / KPIs]

[Counter means the simple counting of the specific processes (e.g. packet number counter, number of connection request counter, …)

[KPI means the representation of the performance metrics, which is the equation of counters per specific time period or any other performance representation]

| Counter | Description | Units | Maximum Value | Action taken when maximum value reached |
| --- | --- | --- | --- | --- |

- KPI #1 – [Name]

[Provide the KPI calculation method, explain the desired result of the KPI graph or other way of the representation]

- KPI #2 [Name]

TBD

# Performance Requirements

[This section should specify the maximum performance capabilities required by the system for the specific feature: e.g. routing table size, maximum queues size, maximum # of queues. It should explain the functionality when the system exceeds the limit: e.g. packet drop, alarm, reset, …]

# Performance Requirement – P0001, [TITLE]

TBD

# Hard Ware Requirements

[To provide HW dependency requirement. E.g. feature requires synchronization need to have PPS with XXX accuracy, protected memory space for the Security Private Key]

# HW Requirement – H0001, [TITLE]

TBD

# Feature Inter-Dependency Requirements

[To explain the possible feature impact with other features of the system]

# Dependency Requirement – D0001, [TITLE]

TBD

# References

TBD

# Glossary

| Term | Description |
| --- | --- |
| foam | Complete GEN2 system, including 4 pillars: lightning, transport, smart city, access and may be more. The system is integrated within the street lightning enclosure. |
| GEN2 | 2nd Generation Juganu system, comprising broadband connectivity of 2gbps and creating novel “foam” paradigm. 2nd GEN refers to the foam transport system, which is based on 802.11ac technology integrated with proprietary Juganu technology |
| foam node | Single lightning unit, comprising 802.11 AP and STA functions. In addition it includes various application capabilities (e.g. cameras, sensors, broadband/cellular/ WiFi small cells, RSU, GPS, …) |
| foam zone or bubble | Group of foam nodes connected between them and creating transport connectivity topology. Foam Zone should comprise at least one POP foam node to connect to the transport backbone. Foam Zone/Bubble may comprise 10s to 100s foam nodes per each |
| Foam globbule | Group of Foam Bubbles typically connected to the backbone from the single aggregation point (e.g. single tower using 802.11ad links/60Ghz) |
| Px (P1, P2, P3) – “Phoenix” P1 & P2 – Campus system P3 – City-wide system | Px refers to the phases of the foam GEN system. Currently defined three phases (Phoenix 1, 2 and 3) - P1 – foam node enclosure +3rd party HW modules, which fits P2 Juganu HW radio systems. GA target: end 2019 - P2 – foam system for limited deployment (20-30 foam nodes per foam bubble). GA target: 2H2020 - P3 – large scale foam system (may reach 200-500 foam nodes per foam bubble). P3 will use smart antenna techniques to achieve high performance and new lightning enclosure. GA target: 2H2021 |
| Foam transport | It is communication using 802.11 technology between the nodes enabling broadband (~2gbps) connectivity between the foam nodes and integrated “applications” toward the backbone cloud/servers/internet access and etc. Foam transport can be used in addition as pier to pier traffic for various foam applications (e.g. connected cars) |
| Transport AP (or Parent) | 802.11ac/ax Access Point functionality (used as BS- Base Station) of the foam transport system. Each foam node comprises one or 2 APs. AP supports MIMO4x4 communicating with STA using SU-MIMO. 802.11 Parent can be OMNI antenna (in P2 system) or Sectorized (Beamswitching) antenna (in P3 system and above) |
| Transport STA (or Child) | 802.11ac/ax MIMO4x4 Station functionality of the foam transport system. Each foam node comprises one STA. 802.11 Child can be OMNI antenna (in P2 system) or Sectorized (Beamswitching) antenna (in P3 system and above) |
| JNET2 Star topology | Point to Multi Point (PTMP) topology, where single AP communicates to many STAs. Star topology is targeted for P1&P2 systems with maximum 1:15 ratio (AP:STA) or higher ratio |
| JNET2 Mixed topology | P1&P2 targeted topology, where star topology can be mixed with chained foam nodes. Mixed topology has “like tree” hierarchical structure, but the main difference from the tree topology, that each Node tries to establish maximum point to multi-point (PTMP) relations AP:STA and minimizing number of hops. |
| JNET2 Tree topology | P3 targeted topology, which creates hierarchical topology between the foam nodes. The root of the tree topology is the POP/CAP node. The targeted tree topology is aligned with daisy connected street lights along the street where the number of hops can be high (e.g. 20 and more). The targeted tree topology will use sectorized antennas and low PTMP ratio compared to “Star” and “Mixed” topologies (e.g. 1:4) |
| JNET2 POP / CAP node | CAP (Central Access Point). The foam CAP node connected to the backbone used as transport Point Of Presence (POP). The POP/CAP node creates 00 foam hierarchy in the tree topology. POP may use 60Ghz PTP link or other communication options (e.g. fiber/SFP, CAT5/Eth…). The major functionality within the POP Node is connecting the external transport world to the Juganu Bubble using different types of transport connectivity protocols (e.g. MPLS, VLAN, L2TP/IPSec, …) CAP/POP Node does not have STA functionality, but may be configured with 2 Transport APs (Parents) and in fact to manage 2 trees topologies (mainly relevant for Sectorized/Beamswitching antenna) |
| JNET2 node | Any Node within the JNET2 bubble excluding POP/CAP Node. Any JNET2 Node consists of one Parent and one Child and functions as traffic transit and/or termination end point within the tree hierarchy. JNET2 Node hierarchy level can be any excluding 00. |
| JNET2 tree Hierarchy XX | JNET2 hierarchy defined as number of radio hops between the POP and the foam node. For example Hierarchy 05 is equivalent to the five hops between the POP and the JNET2 node. |
| foam Application | Any user of the foam transport system defined as the “ foam Application”, Examples for the “foam Application”: cameras, sensors, broadband access, Public WiFi, C-V2X RSU, …. The foam transport should enable proper connectivity requirements per each “ foam application”, which refers to parameters such as Networking (e.g. VLAN, VPN, …), QOS, Security and etc. |
| Foam Application Domain | Identifies the logical transport domain between the applications (or end users) placed and/or connected to the JNET2 node and the Service Provider/Operator Entity. Each such domain manages own IP space, Management, Security and etc (e.g. NMS, DHCP, AAA, …). JNET2 transport should enable connectivity services within the domain, but avoid any connectivity between the domains. Such connectivity can be directed between the Service Provider network / Router and the application end points (within the JNET2 nodes) over tunnels as End to End (E2E) or Peer-to-Peer. |
| Peer to Peer intra-domain service | JNET2 enables Peer-to-Peer traffic within the domain for chosen applications, meaning the JNET2 may deliver IP packets between the application end-points within the domain avoiding back & forward transportation via the domain Core Network |
| Per Domain entities | - AAA/Radius Server/Authentication Server – Authentication, Authorization Accounting Server, used for the security purposes to authenticate the end points (e.g. JNET2 Nodes, IP Cameras, WiFi Users, …). AAA manages and delivers the security keys, which are used for authencity and traffic encryption. Not all AAA functions can be relevant per each domain (e.g. Accounting) - DHCP – used for IP address allocation and management - NMS – Management system. It can be that NMS is new (provided by Juganu) or existing (in this case Juganu solution should enable integration with such system using common protocols (e.g. TR69, SNMP, NetCONF, …) - Others: e.g. GW routers, Servers, … |
| Frequency Bandwidth (BW) | The width of the channel (in MHz) the Tx/Rx communicates |
| Frequency Reuse (1, 2, …) | Frequency reuse refers to the channel reuse per hop in the JNET2 system. For instance Reuse 1 means that each hop uses same frequency channel (f1), Reuse 2 means that one set of hops will use f1 channel and other set of hops will use f2 channel. As target on P1 & P2 with OMNI antenna the system will support Reuse 2 (e.g. BW=80Mhz/hop) and higher, while in P3 the target is Frequency Reuse 1 (e.g. BW=160MHz/hop) |